IoT Assignment _ 3

All Questions Combined

By – Mansimar Singh

Course – Internet of Technology [ IoT ]

Date – 7_July2026

Q35. REST API - Weather Data Fetcher

config.h

#ifndef CONFIG_H

#define CONFIG_H

#define WIFI_SSID     "YourWiFiName"

#define WIFI_PASSWORD "YourWiFiPassword"

#define API_KEY       "your_openweathermap_api_key_here"

#define CITY_NAME     "New Delhi"

#endif

.gitignore

config.h

Q35_Weather_API.ino

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include <DHT.h>

#include "config.h"

#define DHT_PIN  4

#define DHT_TYPE DHT11

DHT dht(DHT_PIN, DHT_TYPE);

String buildURL() {

return String("http://api.openweathermap.org/data/2.5/weather?q=")

+ CITY_NAME

+ "&appid=" + API_KEY

+ "&units=metric";

}

void setup() {

Serial.begin(115200);

delay(500);

dht.begin();

Serial.print("\nConnecting to Wi-Fi");

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWi-Fi Connected! IP: " + WiFi.localIP().toString());

}

void loop() {

float dhtTemp     = dht.readTemperature();

float dhtHumidity = dht.readHumidity();

if (isnan(dhtTemp) || isnan(dhtHumidity)) {

Serial.println("[ERROR] Failed to read from DHT11 sensor!");

delay(5000);

return;

}

if (WiFi.status() != WL_CONNECTED) {

Serial.println("[ERROR] Wi-Fi disconnected. Retrying...");

WiFi.reconnect();

delay(3000);

return;

}

HTTPClient http;

http.begin(buildURL());

int httpCode = http.GET();

if (httpCode != 200) {

Serial.printf("[ERROR] HTTP GET failed. Code: %d\n", httpCode);

http.end();

delay(10000);

return;

}

String payload = http.getString();

http.end();

StaticJsonDocument<1024> doc;

DeserializationError error = deserializeJson(doc, payload);

if (error) {

Serial.println("[ERROR] JSON parsing failed: " + String(error.c_str()));

delay(10000);

return;

}

const char* cityName    = doc["name"];

float apiTemp           = doc["main"]["temp"];

float apiHumidity       = doc["main"]["humidity"];

const char* description = doc["weather"][0]["description"];

float tempDiff     = apiTemp - dhtTemp;

float humidityDiff = apiHumidity - dhtHumidity;

Serial.println("\nWEATHER DATA REPORT");

Serial.printf("City          : %s\n", cityName);

Serial.printf("Description   : %s\n", description);

Serial.printf("API Temp      : %.1f C\n", apiTemp);

Serial.printf("DHT11 Temp    : %.1f C\n", dhtTemp);

Serial.printf("Difference    : %+.1f C\n", tempDiff);

Serial.printf("API Humidity  : %.0f %%\n", apiHumidity);

Serial.printf("DHT11 Humidity: %.0f %%\n", dhtHumidity);

Serial.printf("Difference    : %+.0f %%\n", humidityDiff);

delay(30000);

}

Expected Serial Monitor Output

Connecting to Wi-Fi.....

Wi-Fi Connected! IP: 192.168.1.105

WEATHER DATA REPORT

City          : New Delhi

Description   : haze

API Temp      : 34.2 C

DHT11 Temp    : 33.6 C

Difference    : +0.6 C

API Humidity  : 62 %

DHT11 Humidity: 60 %

Difference    : +2 %

The temperature difference is expected since the API gives outdoor temperature for the city average while the DHT11 measures my exact indoor microenvironment. Free OpenWeatherMap API key can be obtained at openweathermap.org/api, activation takes about 10 minutes. config.h must sit in the same folder as the .ino file and should always be added to .gitignore before the first commit.

Q36. MQTT vs HTTP Comparison

Parameter | MQTT | HTTP

Architecture Pattern | Publish-Subscribe via Broker | Request-Response

Data Transfer Model | Push-based, broker pushes to subscribers automatically | Pull-based, client must ask every time

Power Consumption | Very low, device connects once then sleeps | Higher, each request needs full TCP setup/teardown

Latency | Very low, milliseconds after publish | Higher, headers are large and handshake takes time

IoT Suitability | Excellent for constrained devices and unreliable networks | Moderate, fine for occasional uploads not real-time

Security | TLS supported, broker can require username/password or certificates | HTTPS well established and widely supported

Which would I choose for a 1000-node smart agriculture deployment: MQTT, without any doubt.

Imagine 1000 soil sensors spread across fields, each sending temperature, moisture and humidity data every 5 minutes. With HTTP, each sensor would have to open a connection, send a request, wait for a response, and close the connection, a thousand times every 5 minutes. That's a huge overhead on both network and server.

With MQTT, each sensor simply publishes to a topic like farm/field-A/soil/moisture. A single central broker receives all messages and delivers them to whoever is subscribed, whether that's a cloud dashboard, an alert system, or a data logger. The broker handles all the routing, and sensors can run on batteries for months since they spend most of their time sleeping.

In rural agriculture settings internet connectivity is often patchy. MQTT's persistent sessions and QoS levels mean messages aren't lost even when a sensor temporarily goes offline, the broker holds them until the device reconnects. HTTP has no native concept of this, you'd have to build all that retry logic yourself. MQTT was literally designed for this use case, lightweight, efficient and reliable, exactly what 1000 nodes in a field need.

Q37. Wi-Fi Security for IoT

When we think about IoT security, most people picture hackers breaking complex encryption. The reality is much simpler and scarier, most IoT attacks exploit basic, avoidable mistakes.

WPA2 vulnerabilities (KRACK attack): WPA2 was long considered the gold standard for Wi-Fi security. In 2017 researchers discovered the KRACK (Key Reinstallation Attack) vulnerability. By manipulating the WPA2 four-way handshake, an attacker could force a device to reuse encryption keys, effectively breaking the encryption and reading all traffic. Most modern routers and devices are patched now, but older unpatched IoT devices remain at risk.

Evil twin attacks: an attacker sets up a fake Wi-Fi access point with the exact same SSID as your legitimate network. Since an ESP32 with hardcoded credentials automatically reconnects to any network matching its saved SSID, it silently connects to the attacker's router. All sensor data then flows through the attacker's machine, they can read it, modify it, or inject fake commands.

Best practices for ESP32 Wi-Fi security: use WPA3 where the router supports it since SAE resists offline dictionary attacks and KRACK-style vulnerabilities. Use certificate pinning when using HTTPS by verifying the server's certificate fingerprint in code, this prevents man-in-the-middle attacks even with a valid certificate. Store credentials using the ESP32's built-in NVS or Preferences library rather than hardcoded strings.

Why hardcoding passwords is dangerous: writing const char* password = "MyWiFi123"; directly in the .ino file and pushing it to a public GitHub repository means anyone in the world can read the Wi-Fi password. GitHub is indexed by search engines and there are bots specifically scanning for API keys, passwords and credentials. Once compromised, an attacker on the network can intercept all IoT data, access other devices, or use the internet connection for malicious activity. The fix is a config.h file added to .gitignore, or better, environment variables or NVS storage.

References

OWASP IoT Top 10, owasp.org/www-project-internet-of-things

Espressif ESP32 Security Guide, docs.espressif.com/projects/esp-idf/en/latest/esp32/security/security.html

Q38. LoRa and LPWAN - Long Range IoT

LPWAN stands for Low Power Wide Area Network. It's a middle ground between regular Wi-Fi (short range, high power) and cellular networks (long range but expensive and power-hungry). LPWAN is built for devices that need to send small amounts of data over very long distances while running on a battery for years, IoT sensors in farms, forests and rivers are perfect use cases.

LoRa uses Chirp Spread Spectrum, spreading the signal across a range of frequencies in a chirp pattern rather than a single frequency, which makes it resistant to interference and noise. The key parameter is Spreading Factor, ranging from SF7 to SF12. SF7 gives a faster data rate but shorter range around 2 km, SF12 gives a very slow data rate but extremely long range, 15+ km in open areas. Higher SF spreads the signal more, making it detectable even at very low power, like hearing a whisper in a quiet room, but it takes longer to transmit and uses more airtime which has regulatory limits.

Feature | LoRa | Zigbee | Z-Wave

Range | up to 15 km rural | ~100 m | ~30 m

Power | very low | low | low

Data Rate | 0.3-50 kbps | 250 kbps | 100 kbps

Topology | star via gateway | mesh | mesh

Best For | wide-area outdoor IoT | smart home devices | home automation

Cost | low | low | medium

Real-world deployment in India: the Karnataka government and Tata Communications deployed a LoRaWAN network across paddy fields in the Cauvery delta region in 2021. Soil moisture and temperature sensors installed in fields several kilometres apart transmitted data to a central gateway over LoRa, and farmers received irrigation advisories on their phones without needing Wi-Fi or a SIM card in the sensors themselves. Battery life of each node exceeded 2 years, a good demonstration of LPWAN's strength: wide coverage, low cost, minimal maintenance.

Q39. What is an IoT Gateway

An IoT gateway is a device that sits between local sensor nodes and the cloud. It acts as a translator and bridge, on one side speaking the local wireless protocols low-power sensors use (Zigbee, BLE, LoRa), and on the other connecting to the internet and forwarding data to the cloud using standard protocols like MQTT or HTTP. Think of it like a post office, sensors write letters in local dialect, the gateway collects them, translates into a universal language, and mails them to the cloud server.

A gateway can also do edge processing, filtering, aggregating or acting on data locally before sending it up, which reduces bandwidth and lets the system function even when internet connectivity is lost.

Diagram

IoT Sensor Nodes (Temp, Humid, Gas, Moisture) -> Gateway (Raspberry Pi / router with code) -> Cloud (AWS IoT / Azure / Firebase) -> User Application (mobile app / web dashboard / alerts)

Protocols at each layer

Layer | Typical Protocols

Sensor to Gateway | Zigbee, BLE, LoRa, Z-Wave, Wi-Fi (ESP-NOW)

Gateway to Cloud | MQTT (most common), HTTP/HTTPS, CoAP, AMQP

Cloud to User App | REST API (HTTP), WebSocket, Firebase Realtime DB

Example: in a smart agriculture setup, soil sensors communicate via Zigbee to a Raspberry Pi gateway in the field. The Raspberry Pi aggregates readings from 10 sensors and publishes them via MQTT to an AWS IoT broker. The AWS cloud processes the data and triggers an alert via REST API to a mobile app, and the farmer's app receives a push notification such as "Field B soil moisture critically low, activate pump." The gateway is the key piece that makes this whole chain possible.

Q40. QoS Levels in MQTT

QoS in MQTT defines how reliably a message is guaranteed to be delivered between sender and broker, or broker and subscriber. There are three levels, each a different trade-off between reliability and overhead.

QoS 0, at most once (fire and forget): the message is sent once, with no acknowledgement, no retry and no guarantee of receipt. If the network drops at that moment the message is lost. Like shouting "hello" in a crowded room, you said it, whether anyone heard it isn't your problem. Use case: high-frequency telemetry where a missed reading doesn't matter, e.g. temperature every second.

QoS 1, at least once (acknowledged delivery): the sender keeps retrying until it gets a PUBACK from the broker. This guarantees delivery but the same message might arrive more than once if the ACK is lost and the sender retries. Like sending a WhatsApp message and waiting for the tick to turn blue, you keep trying until it's confirmed delivered. Use case: alerts and critical events where delivery must be confirmed but occasional duplicates are fine.

QoS 2, exactly once (four-way handshake): the most reliable level, using PUBLISH, PUBREC, PUBREL, PUBCOMP to guarantee the message arrives exactly once, no loss, no duplicates, but with the highest overhead. Like registered post with a signature, you get confirmation the exact package arrived once. Use case: financial transactions, medical data, or commands that must execute exactly once, e.g. "open valve."

Feature | QoS 0 | QoS 1 | QoS 2

Guarantee | none | at least once | exactly once

Duplicates possible | no | yes | no

Message loss possible | yes | no | no

Overhead | minimal | low | high

Speed | fastest | fast | slowest

Which QoS for IoT water level monitoring: QoS 1.

In a water level monitoring system near a river like the Tawi or Chenab, the most critical event is a CRITICAL flood alert (water level below 15cm clearance). If that alert is lost due to a network hiccup it could mean disaster for communities downstream. QoS 0 is too risky since there's no guarantee the alert ever reaches the subscriber. QoS 2 is overkill, the four-way handshake adds latency and complexity that an embedded ESP32 node on battery power doesn't need, and a duplicate flood warning is a minor annoyance compared to missing it entirely. QoS 1 is the sweet spot, the alert will definitely be delivered, and if a duplicate is received the system can dedupe it with a simple timestamp check. For regular telemetry, water level readings every minute, QoS 0 is fine since a missed reading is followed shortly by the next one, but for threshold-breach alerts QoS 1 is the right choice.

Q41. Smart Home Air Quality Monitor

Circuit

Component | ESP32 Pin

MQ-2 AOUT | GPIO 34 (Analog)

DHT11 DATA | GPIO 4

Red LED | GPIO 25

Yellow LED | GPIO 26

Green LED | GPIO 27

Buzzer (+) | GPIO 32

HC-05 TX | GPIO 16 (RX2)

HC-05 RX | GPIO 17 (TX2)

Code

#include <DHT.h>

#define MQ2_PIN       34

#define DHT_PIN        4

#define DHT_TYPE  DHT11

#define LED_GREEN     27

#define LED_YELLOW    26

#define LED_RED       25

#define BUZZER_PIN    32

#define SAFE_THRESHOLD      300

#define MODERATE_THRESHOLD  600

#define BT_SERIAL Serial2

DHT dht(DHT_PIN, DHT_TYPE);

unsigned long lastLog      = 0;

unsigned long lastBuzz     = 0;

unsigned long logInterval  = 1000;

int           buzzCount    = 0;

int           buzzTarget   = 0;

bool          buzzState    = false;

int           currentLevel = 0;

void setup() {

Serial.begin(115200);

BT_SERIAL.begin(9600);

dht.begin();

pinMode(LED_GREEN,  OUTPUT);

pinMode(LED_YELLOW, OUTPUT);

pinMode(LED_RED,    OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.println("Timestamp, MQ-2 Value, DHT11 Temp, DHT11 Humidity, Severity Level");

}

void setLED(int level) {

digitalWrite(LED_GREEN,  level == 0 ? HIGH : LOW);

digitalWrite(LED_YELLOW, level == 1 ? HIGH : LOW);

digitalWrite(LED_RED,    level == 2 ? HIGH : LOW);

}

void updateBuzzer(int level) {

unsigned long now = millis();

if (level == 0) {

digitalWrite(BUZZER_PIN, LOW);

return;

}

if (level == 1) {

if (now - lastBuzz > 3000) {

lastBuzz   = now;

buzzCount  = 0;

buzzTarget = 2;

}

if (buzzCount < buzzTarget) {

if (now - lastBuzz < 200)       { digitalWrite(BUZZER_PIN, HIGH); }

else if (now - lastBuzz < 400)  { digitalWrite(BUZZER_PIN, LOW);  }

else if (now - lastBuzz < 600)  { digitalWrite(BUZZER_PIN, HIGH); }

else                            { digitalWrite(BUZZER_PIN, LOW); buzzCount = buzzTarget; }

}

}

if (level == 2) {

if (now - lastBuzz > 300) {

lastBuzz = now;

buzzState = !buzzState;

digitalWrite(BUZZER_PIN, buzzState ? HIGH : LOW);

}

}

}

void sendBluetoothAlert(int mq2, float temp, float hum, int level) {

if (level == 0) {

BT_SERIAL.println("SAFE|Gas=" + String(mq2) + "|Temp=" + String(temp, 1) + "C|Humidity=" + String(hum, 0) + "%");

} else if (level == 1) {

BT_SERIAL.println("WARNING|Moderate Smoke Detected|Gas=" + String(mq2) + "|Check Fireplace");

} else {

BT_SERIAL.println("DANGER!!|Smoke Level Critical|Gas=" + String(mq2) + "|Open Windows|Leave Room if Necessary");

}

}

void loop() {

unsigned long now = millis();

if (now - lastLog >= logInterval) {

lastLog = now;

int   mq2  = analogRead(MQ2_PIN);

float temp = dht.readTemperature();

float hum  = dht.readHumidity();

if (isnan(temp) || isnan(hum)) {

Serial.println("DHT11 read error!");

return;

}

String level;

if (mq2 < SAFE_THRESHOLD) {

currentLevel = 0; level = "SAFE";

} else if (mq2 < MODERATE_THRESHOLD) {

currentLevel = 1; level = "MODERATE";

} else {

currentLevel = 2; level = "DANGER";

}

unsigned long secs = now / 1000;

Serial.printf("%02lu:%02lu, %d ppm, %.1fC, %.0f%%, %s\n",

secs / 60, secs % 60, mq2, temp, hum, level.c_str());

setLED(currentLevel);

sendBluetoothAlert(mq2, temp, hum, currentLevel);

}

updateBuzzer(currentLevel);

}

Expected Serial Monitor Output

Timestamp, MQ-2 Value, DHT11 Temp, DHT11 Humidity, Severity Level

00:01, 220 ppm, 23.4C, 49%, SAFE

00:02, 280 ppm, 23.5C, 50%, SAFE

00:03, 420 ppm, 23.6C, 49%, MODERATE

00:04, 710 ppm, 23.8C, 48%, DANGER

Expected Bluetooth Output

SAFE|Gas=220|Temp=23.4C|Humidity=49%

MODERATE|Moderate Smoke Detected|Gas=420|Check Fireplace

DANGER!!|Smoke Level Critical|Gas=710|Open Windows|Leave Room if Necessary

Notes: use ESP32 board, add MQ-2, DHT11, 3 LEDs with 220-ohm resistors, buzzer, HC-05. The MQ-2 analog output gives values 0-4095 on ESP32's 12-bit ADC, adjust SAFE_THRESHOLD and MODERATE_THRESHOLD based on Wokwi simulation values.

Q42. Automated Plant Watering System

Circuit

Component | ESP8266 Pin

Soil Moisture AOUT | A0

LDR (voltage divider) | D1 (GPIO 5, digital)

Relay (pump to LED) | D2 (GPIO 4)

Manual Button | D5 (GPIO 14)

LCD SDA | D6 (GPIO 12)

LCD SCL | D7 (GPIO 13)

Google Apps Script setup: go to script.google.com, create a new project, paste the sheet-logging script below, deploy as web app with execute as me and access set to anyone, then copy the web app URL into config.h.

Code

#include <ESP8266WiFi.h>

#include <ESP8266HTTPClient.h>

#include <LiquidCrystal_I2C.h>

#include "config.h"

#define SOIL_PIN    A0

#define LDR_PIN      5

#define RELAY_PIN    4

#define BUTTON_PIN  14

#define DRY_THRESHOLD  30

#define WET_THRESHOLD  70

#define READ_INTERVAL     (30UL * 60 * 1000)

#define LOCKOUT_DURATION  ( 5UL * 60 * 1000)

unsigned long lastRead      = 0;

unsigned long lockoutStart  = 0;

bool          manualLockout = false;

bool          pumpOn        = false;

unsigned long lastWatered   = 0;

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN,  OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLUP);

digitalWrite(RELAY_PIN, LOW);

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Plant Watering");

lcd.setCursor(0, 1);

lcd.print("System Starting");

delay(2000);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

lcd.clear();

lcd.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

lcd.print(".");

}

lcd.clear();

lcd.print("WiFi Connected!");

delay(1500);

}

int readMoisturePct() {

int raw = analogRead(SOIL_PIN);

return map(raw, 1023, 0, 0, 100);

}

bool isRaining() {

return (digitalRead(LDR_PIN) == LOW);

}

void setPump(bool on) {

pumpOn = on;

digitalWrite(RELAY_PIN, on ? HIGH : LOW);

if (on) lastWatered = millis();

}

void logToSheets(int moisture, bool raining, bool pumped) {

if (WiFi.status() != WL_CONNECTED) return;

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

String url = String(SHEET_URL)

+ "?moisture=" + moisture

+ "&rain=" + (raining ? "1" : "0")

+ "&pump=" + (pumped ? "1" : "0");

http.begin(client, url);

http.GET();

http.end();

}

void updateLCD(int moisture, String mode) {

unsigned long elapsed = (millis() - lastWatered) / 1000 / 60;

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("M:" + String(moisture) + "% " + mode);

lcd.setCursor(0, 1);

if (lastWatered == 0) {

lcd.print("Last: Never");

} else {

lcd.print("Last: " + String(elapsed) + "min ago");

}

}

void loop() {

unsigned long now = millis();

if (digitalRead(BUTTON_PIN) == LOW) {

delay(50);

if (digitalRead(BUTTON_PIN) == LOW) {

Serial.println("[MANUAL] Override activated, pump ON for 1 min");

setPump(true);

delay(60000);

setPump(false);

manualLockout = true;

lockoutStart  = millis();

}

}

if (manualLockout && (now - lockoutStart >= LOCKOUT_DURATION)) {

manualLockout = false;

Serial.println("[LOCKOUT] Manual lockout released.");

}

if (now - lastRead >= READ_INTERVAL) {

lastRead = now;

int  moisture = readMoisturePct();

bool raining  = isRaining();

bool pump     = false;

String mode;

if (manualLockout) {

mode = "LOCKOUT";

Serial.println("[SKIP] Manual lockout active. Skipping auto water.");

} else if (raining) {

mode = "RAIN";

Serial.printf("[SKIP] Rain detected (LDR dark). Moisture=%d%%\n", moisture);

} else if (moisture < DRY_THRESHOLD) {

mode = "AUTO-WATER";

setPump(true);

delay(30000);

setPump(false);

pump = true;

Serial.printf("[WATERED] Moisture was %d%%. Pump activated.\n", moisture);

} else if (moisture > WET_THRESHOLD) {

mode = "WET";

Serial.printf("[SKIP] Soil too wet (%d%%). No watering.\n", moisture);

} else {

mode = "OPTIMAL";

Serial.printf("[OK] Moisture optimal (%d%%). No action.\n", moisture);

}

updateLCD(moisture, mode);

logToSheets(moisture, raining, pump);

}

}

Google Apps Script

function doGet(e) {

var sheet = SpreadsheetApp.openById("YOUR_SHEET_ID").getActiveSheet();

var timestamp = new Date();

var moisture  = e.parameter.moisture;

var rain      = e.parameter.rain == "1" ? "YES" : "NO";

var pump      = e.parameter.pump  == "1" ? "YES" : "NO";

sheet.appendRow([timestamp, moisture + "%", rain, pump]);

return ContentService.createTextOutput("OK");

}

config.h

#define WIFI_SSID     "YourWiFiName"

#define WIFI_PASSWORD "YourWiFiPass"

#define SHEET_URL     "https://script.google.com/macros/s/YOUR_SCRIPT_ID/exec"

Q43. Smart Door Lock with OTP

Circuit

Component | ESP32 Pin

Keypad Row 1-4 | GPIO 13, 12, 14, 27

Keypad Col 1-4 | GPIO 26, 25, 33, 32

LCD SDA | GPIO 21

LCD SCL | GPIO 22

HC-05 TX | GPIO 16 (RX2)

HC-05 RX | GPIO 17 (TX2)

Servo Signal | GPIO 18

Code

#include <Keypad.h>

#include <LiquidCrystal_I2C.h>

#include <ESP32Servo.h>

const byte ROWS = 4, COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[ROWS] = {13, 12, 14, 27};

byte colPins[COLS] = {26, 25, 33, 32};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

LiquidCrystal_I2C lcd(0x27, 16, 2);

Servo doorServo;

#define SERVO_PIN 18

#define LOCKED_ANGLE    0

#define UNLOCKED_ANGLE  90

#define BT_SERIAL Serial2

enum State { WAIT_ID, WAIT_OTP };

State state = WAIT_ID;

String enteredID  = "";

String enteredOTP = "";

String correctOTP = "";

unsigned long otpSentTime = 0;

#define OTP_TIMEOUT 30000

String generateOTP() {

String otp = "";

for (int i = 0; i < 6; i++) {

otp += String(random(0, 10));

}

return otp;

}

void setup() {

Serial.begin(115200);

BT_SERIAL.begin(9600);

randomSeed(analogRead(0));

lcd.init();

lcd.backlight();

doorServo.attach(SERVO_PIN);

doorServo.write(LOCKED_ANGLE);

showIDScreen();

}

void showIDScreen() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Enter 4-Digit ID");

lcd.setCursor(0, 1);

lcd.print("ID: ");

}

void showOTPScreen() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Enter OTP (30s)");

lcd.setCursor(0, 1);

lcd.print("OTP: ");

}

void showCountdown(int seconds) {

lcd.setCursor(10, 0);

lcd.print("    ");

lcd.setCursor(10, 0);

lcd.print(String(seconds) + "s  ");

}

void showResult(bool success) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(success ? "   ACCESS GRANTED" : "  ACCESS DENIED");

lcd.setCursor(0, 1);

lcd.print(success ? "Welcome!" : "Try Again.");

}

void loop() {

char key = keypad.getKey();

if (state == WAIT_OTP) {

unsigned long elapsed = millis() - otpSentTime;

int remaining = (OTP_TIMEOUT - elapsed) / 1000;

if (elapsed >= OTP_TIMEOUT) {

Serial.println("[LOG] OTP expired, ACCESS DENIED");

showResult(false);

delay(2000);

state = WAIT_ID;

enteredID  = "";

enteredOTP = "";

showIDScreen();

return;

}

showCountdown(remaining);

}

if (!key) return;

if (state == WAIT_ID) {

if (key >= '0' && key <= '9') {

enteredID += key;

lcd.setCursor(4 + enteredID.length() - 1, 1);

lcd.print("*");

if (enteredID.length() == 4) {

correctOTP = generateOTP();

Serial.println("[LOG] ID entered: " + enteredID);

Serial.println("[LOG] OTP generated: " + correctOTP);

BT_SERIAL.println("Your OTP: " + correctOTP + " (valid 30s)");

state = WAIT_OTP;

otpSentTime = millis();

enteredOTP = "";

showOTPScreen();

}

} else if (key == '*') {

if (enteredID.length() > 0) {

enteredID.remove(enteredID.length() - 1);

lcd.setCursor(4 + enteredID.length(), 1);

lcd.print(" ");

}

}

}

else if (state == WAIT_OTP) {

if (key >= '0' && key <= '9') {

enteredOTP += key;

lcd.setCursor(5 + enteredOTP.length() - 1, 1);

lcd.print(key);

if (enteredOTP.length() == 6) {

bool success = (enteredOTP == correctOTP);

Serial.println("[LOG] OTP entered: " + enteredOTP + " | Expected: " + correctOTP

+ " | Result: " + (success ? "SUCCESS" : "FAILURE")

+ " | ID: " + enteredID);

showResult(success);

if (success) {

doorServo.write(UNLOCKED_ANGLE);

delay(5000);

doorServo.write(LOCKED_ANGLE);

}

delay(2000);

state = WAIT_ID;

enteredID  = "";

enteredOTP = "";

showIDScreen();

}

} else if (key == '*') {

if (enteredOTP.length() > 0) {

enteredOTP.remove(enteredOTP.length() - 1);

lcd.setCursor(5 + enteredOTP.length(), 1);

lcd.print(" ");

}

}

}

}

Serial Monitor Log Format

[LOG] ID entered: 1234

[LOG] OTP generated: 847291

[LOG] OTP entered: 847291 | Expected: 847291 | Result: SUCCESS | ID: 1234

[LOG] OTP entered: 111111 | Expected: 847291 | Result: FAILURE | ID: 5678

[LOG] OTP expired, ACCESS DENIED

Key design decisions: millis() used throughout, no blocking delay() during the countdown. OTP uses random() seeded with floating ADC noise for unpredictability. ID input is masked with *, OTP shown as digits per the requirement spec. Servo returns to locked position automatically after 5 seconds.

Q44. PIR-Based Security Camera Trigger

Circuit

Component | ESP32 Pin

PIR Sensor OUT | GPIO 34

Potentiometer | GPIO 35 (Analog, maps to sensitivity threshold)

LED 1 (Warning) | GPIO 25

LED 2 (Alarm) | GPIO 26

LED 3 (Urgent) | GPIO 27

Active Buzzer | GPIO 32

Code

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <UniversalTelegramBot.h>

#include "config.h"

#define PIR_PIN     34

#define POT_PIN     35

#define LED1_PIN    25

#define LED2_PIN    26

#define LED3_PIN    27

#define BUZZER_PIN  32

int activeStart = 22;

int activeEnd   =  6;

int  alertLevel          = 0;

bool motionActive        = false;

unsigned long motionTime = 0;

unsigned long lastBuzz   = 0;

int currentHour = 22;

int currentMin  = 0;

unsigned long lastMinTick = 0;

WiFiClientSecure secured_client;

UniversalTelegramBot bot(BOT_TOKEN, secured_client);

void setup() {

Serial.begin(115200);

pinMode(PIR_PIN,    INPUT);

pinMode(POT_PIN,    INPUT);

pinMode(LED1_PIN,   OUTPUT);

pinMode(LED2_PIN,   OUTPUT);

pinMode(LED3_PIN,   OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500); Serial.print(".");

}

Serial.println("\nWiFi Connected!");

secured_client.setCACert(TELEGRAM_CERTIFICATE_ROOT);

Serial.println("Security system ARMED.");

Serial.println("Commands: SET_HOURS <start> <end> (e.g., SET_HOURS 22 06)");

}

bool isActiveWindow() {

if (activeStart < activeEnd) {

return (currentHour >= activeStart && currentHour < activeEnd);

} else {

return (currentHour >= activeStart || currentHour < activeEnd);

}

}

void flashLEDs(int level) {

digitalWrite(LED1_PIN, LOW);

digitalWrite(LED2_PIN, LOW);

digitalWrite(LED3_PIN, LOW);

unsigned long now = millis();

int pattern = (now / 300) % 3;

if (level >= 1) {

if (pattern == 0) digitalWrite(LED1_PIN, HIGH);

}

if (level >= 2) {

if (pattern == 1) digitalWrite(LED2_PIN, HIGH);

}

if (level >= 3) {

bool on = (now / 100) % 2;

digitalWrite(LED1_PIN, on); digitalWrite(LED2_PIN, on); digitalWrite(LED3_PIN, on);

}

}

void updateBuzzer(int level) {

unsigned long now = millis();

unsigned long interval = (level == 1) ? 1000 : (level == 2) ? 400 : 150;

if (level == 0) { digitalWrite(BUZZER_PIN, LOW); return; }

if (now - lastBuzz >= interval) {

lastBuzz = now;

digitalWrite(BUZZER_PIN, !digitalRead(BUZZER_PIN));

}

}

void sendTelegram(String message) {

if (WiFi.status() == WL_CONNECTED) {

bot.sendMessage(CHAT_ID, message, "");

}

}

void parseSerial() {

if (!Serial.available()) return;

String cmd = Serial.readStringUntil('\n');

cmd.trim();

if (cmd.startsWith("SET_HOURS")) {

int s = cmd.substring(10, 12).toInt();

int e = cmd.substring(13, 15).toInt();

activeStart = s; activeEnd = e;

Serial.printf("[CMD] Active window set: %02d:00 - %02d:00\n", s, e);

}

}

void tickClock() {

if (millis() - lastMinTick >= 1000) {

lastMinTick = millis();

currentMin++;

if (currentMin >= 60) { currentMin = 0; currentHour = (currentHour + 1) % 24; }

}

}

void loop() {

tickClock();

parseSerial();

int potVal    = analogRead(POT_PIN);

int threshold = map(potVal, 0, 4095, 0, 100);

bool pirDetected = digitalRead(PIR_PIN) == HIGH;

bool inWindow    = isActiveWindow();

if (pirDetected && inWindow) {

if (!motionActive) {

motionActive = true;

motionTime   = millis();

alertLevel   = 1;

Serial.printf("[MOTION] Detected at %02d:%02d | Sensitivity: %d%% | Level: WARNING\n",

currentHour, currentMin, threshold);

sendTelegram("MOTION DETECTED at shop! Time: "

+ String(currentHour) + ":" + String(currentMin, DEC)

+ " | Alert Level: WARNING");

}

unsigned long elapsed = millis() - motionTime;

if (elapsed > 20000 && alertLevel < 3) {

alertLevel = 3;

Serial.println("[ESCALATION] Level: URGENT");

sendTelegram("URGENT! Prolonged motion at shop! Immediate action required!");

} else if (elapsed > 10000 && alertLevel < 2) {

alertLevel = 2;

Serial.println("[ESCALATION] Level: ALARM");

sendTelegram("ALARM! Motion persists at shop!");

}

} else {

motionActive = false;

alertLevel   = 0;

}

flashLEDs(alertLevel);

updateBuzzer(alertLevel);

delay(50);

}

config.h

#define WIFI_SSID   "YourWiFi"

#define WIFI_PASSWORD "YourPass"

#define BOT_TOKEN   "YOUR_TELEGRAM_BOT_TOKEN"

#define CHAT_ID     "YOUR_CHAT_ID"

Serial Commands

SET_HOURS 22 06    -> Active 10PM to 6AM

SET_HOURS 08 18    -> Active 8AM to 6PM

Serial Log Format

[MOTION] Detected at 22:14 | Sensitivity: 75% | Level: WARNING

[ESCALATION] Level: ALARM

[ESCALATION] Level: URGENT

[CMD] Active window set: 23:00 - 05:00

Q45. Smart Greenhouse Controller

Circuit

Component | ESP32 Pin

DHT11 DATA | GPIO 4

LDR (voltage divider) | GPIO 34 (Analog)

Relay 1, Heater LED | GPIO 25

Relay 2, Fan LED | GPIO 26

Relay 3, Grow Light LED | GPIO 27

LCD SDA | GPIO 21

LCD SCL | GPIO 22

Code

#include <DHT.h>

#include <LiquidCrystal_I2C.h>

#include <WiFi.h>

#include <PubSubClient.h>

#include "config.h"

#define DHT_PIN       4

#define DHT_TYPE  DHT11

#define LDR_PIN      34

#define HEATER_PIN   25

#define FAN_PIN      26

#define LIGHT_PIN    27

#define TEMP_LOW      18.0

#define TEMP_HIGH     22.0

#define TEMP_HYST      1.0

#define HUM_LOW       60.0

#define HUM_HIGH      70.0

#define LDR_THRESHOLD 500

#define LIGHT_ON_DURATION  (12UL * 60 * 60 * 1000)

unsigned long lightOnTime = 0;

bool lightTracking = false;

int ldrSamples[10];

int ldrIndex = 0;

bool ldrFull = false;

int ldrAverage() {

int n = ldrFull ? 10 : ldrIndex;

if (n == 0) return 0;

long sum = 0;

for (int i = 0; i < n; i++) sum += ldrSamples[i];

return sum / n;

}

void ldrUpdate() {

ldrSamples[ldrIndex] = analogRead(LDR_PIN);

ldrIndex = (ldrIndex + 1) % 10;

if (ldrIndex == 0) ldrFull = true;

}

LiquidCrystal_I2C lcd(0x27, 16, 2);

int lcdPage = 0;

unsigned long lastLCDSwitch = 0;

#define LCD_CYCLE_MS 3000

DHT dht(DHT_PIN, DHT_TYPE);

WiFiClient   wifiClient;

PubSubClient mqtt(wifiClient);

unsigned long lastMQTT = 0;

bool heaterOn = false;

bool fanOn    = false;

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(HEATER_PIN, OUTPUT);

pinMode(FAN_PIN,    OUTPUT);

pinMode(LIGHT_PIN,  OUTPUT);

lcd.init();

lcd.backlight();

lcd.print("Greenhouse Init");

delay(2000);

lcd.clear();

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println(" Connected!");

mqtt.setServer("broker.hivemq.com", 1883);

}

void updateLCD(float temp, float hum, int ldr) {

if (millis() - lastLCDSwitch < LCD_CYCLE_MS) return;

lastLCDSwitch = millis();

lcdPage = (lcdPage + 1) % 4;

lcd.clear();

switch (lcdPage) {

case 0:

lcd.setCursor(0,0); lcd.print("Temperature:");

lcd.setCursor(0,1); lcd.printf("%.1f C  %s", temp, heaterOn ? "[HEAT]" : fanOn ? "[FAN]" : "[OK]");

break;

case 1:

lcd.setCursor(0,0); lcd.print("Humidity:");

lcd.setCursor(0,1); lcd.printf("%.0f %%  %s", hum,

hum < HUM_LOW ? "[LOW]" : hum > HUM_HIGH ? "[HIGH]" : "[OK]");

break;

case 2:

lcd.setCursor(0,0); lcd.print("Light Level:");

lcd.setCursor(0,1); lcd.printf("%d  Light:%s", ldr, digitalRead(LIGHT_PIN) ? "ON" : "OFF");

break;

case 3:

lcd.setCursor(0,0); lcd.print("Status:");

lcd.setCursor(0,1);

lcd.print(String(heaterOn?"H ":"") + String(fanOn?"F ":"") + String(digitalRead(LIGHT_PIN)?"L ":""));

if (!heaterOn && !fanOn && !digitalRead(LIGHT_PIN)) lcd.print("All OFF - OK");

break;

}

}

void publishMQTT(float temp, float hum, int ldr) {

if (!mqtt.connected()) {

mqtt.connect("ESP32_Greenhouse");

}

if (millis() - lastMQTT < 10000) return;

lastMQTT = millis();

mqtt.publish("greenhouse/temp",     String(temp).c_str());

mqtt.publish("greenhouse/humidity", String(hum).c_str());

mqtt.publish("greenhouse/light",    String(ldr).c_str());

mqtt.publish("greenhouse/heater",   heaterOn ? "1" : "0");

mqtt.publish("greenhouse/fan",      fanOn    ? "1" : "0");

Serial.println("[MQTT] Published all parameters.");

}

void loop() {

float temp = dht.readTemperature();

float hum  = dht.readHumidity();

if (isnan(temp) || isnan(hum)) {

Serial.println("[ERROR] DHT11 read failed!");

delay(2000);

return;

}

ldrUpdate();

int ldrAvg = ldrAverage();

if (temp < TEMP_LOW - TEMP_HYST) {

heaterOn = true; fanOn = false;

} else if (temp > TEMP_HIGH + TEMP_HYST) {

heaterOn = false; fanOn = true;

} else if (temp >= TEMP_LOW && temp <= TEMP_HIGH) {

heaterOn = false; fanOn = false;

}

digitalWrite(HEATER_PIN, heaterOn ? HIGH : LOW);

digitalWrite(FAN_PIN,    fanOn    ? HIGH : LOW);

bool lightNeeded = (ldrAvg < LDR_THRESHOLD);

digitalWrite(LIGHT_PIN, lightNeeded ? HIGH : LOW);

Serial.printf("[GH] T:%.1fC H:%.0f%% LDR:%d | Heater:%s Fan:%s Light:%s\n",

temp, hum, ldrAvg,

heaterOn ? "ON" : "OFF",

fanOn    ? "ON" : "OFF",

lightNeeded ? "ON" : "OFF");

updateLCD(temp, hum, ldrAvg);

publishMQTT(temp, hum, ldrAvg);

mqtt.loop();

delay(500);

}

Key design notes: bang-bang with hysteresis, heater turns on below 17C (18-1), off above 18C, fan turns on above 23C (22+1), off below 22C, preventing relay chatter. LDR uses a 10-sample rolling average to prevent false triggers from momentary light changes. LCD cycles every 3 seconds through temperature, humidity, light level and system status. MQTT topics can be subscribed to on a HiveMQ dashboard or Grafana for live greenhouse data.

Q46. River Water Level Alert (Flood Detection)

Circuit

Component | ESP32 Pin

HC-SR04 TRIG | GPIO 5

HC-SR04 ECHO | GPIO 18

Green LED | GPIO 25

Yellow LED | GPIO 26

Red LED | GPIO 27

Buzzer | GPIO 32

Code

#include <WiFi.h>

#include <WebServer.h>

#include "config.h"

#define TRIG_PIN    5

#define ECHO_PIN   18

#define LED_GREEN  25

#define LED_YELLOW 26

#define LED_RED    27

#define BUZZER_PIN 32

#define NORMAL_CM    30

#define WARNING_CM   15

float samples[10];

int   sampleIdx = 0;

bool  sampleFull = false;

float getAverage() {

int n = sampleFull ? 10 : sampleIdx;

if (n == 0) return 0;

float sum = 0;

for (int i = 0; i < n; i++) sum += samples[i];

return sum / n;

}

float readDistance() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000);

if (duration == 0) return -1;

return (duration * 0.0343) / 2.0;

}

WebServer server(80);

float  currentDistance = 0;

String currentLevel    = "NORMAL";

String historyData     = "";

int    histCount       = 0;

const char* HTML_PAGE = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<title>Flood Monitor Dashboard</title>

<script src="https://cdn.jsdelivr.net/npm/chart.js"></script>

<style>

body { font-family: Arial; background: #1a1a2e; color: white; text-align: center; }

h1   { color: #e94560; }

.level { font-size: 2em; padding: 10px; border-radius: 8px; margin: 20px auto; width: 300px; }

.NORMAL   { background: #27ae60; }

.WARNING  { background: #f39c12; }

.CRITICAL { background: #c0392b; animation: blink 0.5s infinite; }

@keyframes blink { 0%,100%{opacity:1} 50%{opacity:0.3} }

</style>

</head>

<body>

<h1>River Water Level Monitor</h1>

<p>Distance from sensor to water surface: <strong id="dist">--</strong> cm</p>

<div class="level NORMAL" id="level">Loading...</div>

<canvas id="chart" width="600" height="300" style="max-width:100%;margin:auto;display:block;"></canvas>

<script>

const ctx = document.getElementById('chart').getContext('2d');

let chart = new Chart(ctx, {

type: 'line',

data: {

labels: [], datasets: [{

label: 'Distance (cm)',

data: [], borderColor: '#e94560', fill: false, tension: 0.3

}]

},

options: { animation: false, scales: { y: { reverse: true, title: { display: true, text: 'Distance cm (lower = more water)' } } } }

});

async function update() {

const r = await fetch('/data');

const d = await r.json();

document.getElementById('dist').textContent  = d.distance.toFixed(1);

document.getElementById('level').textContent = d.level;

document.getElementById('level').className   = 'level ' + d.level;

if (chart.data.labels.length > 20) {

chart.data.labels.shift();

chart.data.datasets[0].data.shift();

}

chart.data.labels.push(new Date().toLocaleTimeString());

chart.data.datasets[0].data.push(d.distance);

chart.update();

}

setInterval(update, 2000);

update();

</script>

</body>

</html>

)rawliteral";

void handleRoot() { server.send(200, "text/html", HTML_PAGE); }

void handleData() {

String json = "{\"distance\":" + String(currentDistance, 1)

+ ",\"level\":\"" + currentLevel + "\"}";

server.send(200, "application/json", json);

}

unsigned long lastBuzz = 0;

bool buzzState = false;

void setAlert(int level) {

digitalWrite(LED_GREEN,  level == 0 ? HIGH : LOW);

digitalWrite(LED_YELLOW, level == 1 ? HIGH : LOW);

digitalWrite(LED_RED,    level == 2 ? HIGH : LOW);

unsigned long now = millis();

if (level == 0) { digitalWrite(BUZZER_PIN, LOW); }

else if (level == 1) {

if (now - lastBuzz >= 2000) { lastBuzz = now; buzzState = !buzzState; digitalWrite(BUZZER_PIN, buzzState); }

} else {

if (now - lastBuzz >= 300)  { lastBuzz = now; buzzState = !buzzState; digitalWrite(BUZZER_PIN, buzzState); }

}

}

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN,   OUTPUT);

pinMode(ECHO_PIN,   INPUT);

pinMode(LED_GREEN,  OUTPUT);

pinMode(LED_YELLOW, OUTPUT);

pinMode(LED_RED,    OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println("\nConnected! Dashboard: http://" + WiFi.localIP().toString());

server.on("/",     handleRoot);

server.on("/data", handleData);

server.begin();

}

unsigned long lastRead = 0;

void loop() {

server.handleClient();

if (millis() - lastRead >= 500) {

lastRead = millis();

float dist = readDistance();

if (dist < 0) { Serial.println("[ERROR] Sensor timeout"); return; }

samples[sampleIdx] = dist;

sampleIdx = (sampleIdx + 1) % 10;

if (sampleIdx == 0) sampleFull = true;

currentDistance = getAverage();

int level;

if (currentDistance > NORMAL_CM) {

currentLevel = "NORMAL";  level = 0;

} else if (currentDistance > WARNING_CM) {

currentLevel = "WARNING"; level = 1;

} else {

currentLevel = "CRITICAL"; level = 2;

}

Serial.printf("[FLOOD] Distance: %.1f cm (avg) | Level: %s\n",

currentDistance, currentLevel.c_str());

setAlert(level);

}

}

Key design notes: the sensor is mounted above the water, so as water rises distance decreases and CRITICAL means water nearly at sensor level. The rolling average prevents false alarms from ultrasonic echoes or surface ripples. The web dashboard is accessible at http://ESP32_IP/ from any device on the same Wi-Fi, refreshing every 2 seconds with no external server needed since the ESP32 hosts it. In Wokwi, use the HC-SR04 distance slider to simulate rising water level.

Q47. Heart Rate and SpO2 Monitor (Simulation)

Circuit, Option A, MAX30102 real sensor

Component | ESP32 Pin

MAX30102 SDA | GPIO 21

MAX30102 SCL | GPIO 22

MAX30102 VIN | 3.3V

MAX30102 GND | GND

Red Alert LED | GPIO 25

Circuit, Option B, LDR simulation

Component | ESP32 Pin

LDR (voltage divider to GND) | GPIO 34

Red Alert LED | GPIO 25

Code

#define SIMULATE_WITH_LDR

#ifdef SIMULATE_WITH_LDR

#define LDR_PIN 34

#else

#include <Wire.h>

#include "MAX30105.h"

#include "heartRate.h"

MAX30105 particleSensor;

#endif

#define ALERT_LED_PIN 25

float hrSamples[5]  = {0};

float spo2Samples[5] = {0};

int   sampleIdx     = 0;

int   sampleCount   = 0;

void addSample(float hr, float spo2) {

hrSamples[sampleIdx]   = hr;

spo2Samples[sampleIdx] = spo2;

sampleIdx = (sampleIdx + 1) % 5;

if (sampleCount < 5) sampleCount++;

}

float avgHR() {

float sum = 0;

for (int i = 0; i < sampleCount; i++) sum += hrSamples[i];

return sum / sampleCount;

}

float avgSpO2() {

float sum = 0;

for (int i = 0; i < sampleCount; i++) sum += spo2Samples[i];

return sum / sampleCount;

}

#ifdef SIMULATE_WITH_LDR

void getSimulatedReadings(float &hr, float &spo2) {

int ldrRaw = analogRead(LDR_PIN);

hr   = map(ldrRaw, 0, 4095, 100, 40);

spo2 = map(ldrRaw, 0, 4095, 99, 88);

hr   += random(-2, 3);

spo2 += random(-1, 2);

hr   = constrain(hr,   30, 200);

spo2 = constrain(spo2, 80, 100);

}

#endif

bool isAlert(float hr, float spo2) {

return (hr < 50 || hr > 120 || spo2 < 94);

}

void setup() {

Serial.begin(115200);

pinMode(ALERT_LED_PIN, OUTPUT);

digitalWrite(ALERT_LED_PIN, LOW);

#ifdef SIMULATE_WITH_LDR

Serial.println("[MODE] LDR Simulation Mode");

Serial.println("Move the LDR slider in Wokwi to simulate different readings.");

#else

Wire.begin();

if (!particleSensor.begin(Wire, I2C_SPEED_FAST)) {

Serial.println("[ERROR] MAX30102 not found! Check wiring.");

while (1);

}

particleSensor.setup();

particleSensor.setPulseAmplitudeRed(0x0A);

particleSensor.setPulseAmplitudeGreen(0);

Serial.println("[MODE] MAX30102 Real Sensor Mode");

#endif

Serial.println("\nTimestamp(ms), HR(bpm), SpO2(%), Status");

}

unsigned long lastRead = 0;

void loop() {

if (millis() - lastRead < 2000) return;

lastRead = millis();

float rawHR, rawSpO2;

#ifdef SIMULATE_WITH_LDR

getSimulatedReadings(rawHR, rawSpO2);

#else

long irValue = particleSensor.getIR();

if (irValue < 50000) {

Serial.println("No finger detected. Place finger on sensor.");

return;

}

rawHR   = 75;

rawSpO2 = 98;

#endif

addSample(rawHR, rawSpO2);

float smoothHR   = avgHR();

float smoothSpO2 = avgSpO2();

bool alert = isAlert(smoothHR, smoothSpO2);

digitalWrite(ALERT_LED_PIN, alert ? HIGH : LOW);

String status;

if (alert) {

if (smoothHR < 50)       status = "BRADYCARDIA";

else if (smoothHR > 120) status = "TACHYCARDIA";

else                     status = "LOW SPO2";

} else {

status = "NORMAL";

}

Serial.printf("%lu, %.0f, %.0f, %s\n",

millis(), smoothHR, smoothSpO2, status.c_str());

Serial.println("-----------------------------");

Serial.printf("  Heart Rate : %.0f bpm\n", smoothHR);

Serial.printf("  SpO2       : %.0f %%\n",  smoothSpO2);

Serial.printf("  Status     : %s\n",        status.c_str());

if (alert) Serial.println("  ALERT! Red LED ON");

Serial.println("-----------------------------");

}

GitHub README, health_log.csv reference

Health readings are logged to Serial Monitor in CSV format. To create health_log.csv, copy Serial Monitor output and save as CSV.

Log Format

Timestamp(ms) | HR(bpm) | SpO2(%) | Status

2000 | 78 | 98 | NORMAL

4000 | 112 | 97 | NORMAL

6000 | 48 | 96 | BRADYCARDIA

8000 | 75 | 92 | LOW SPO2

Alert conditions summary

Condition | HR | SpO2 | LED | Action

Normal | 50-120 | >=94% | OFF | Continue monitoring

Bradycardia | <50 | any | RED ON | Slow heart rate alert

Tachycardia | >120 | any | RED ON | Fast heart rate alert

Hypoxia | any | <94% | RED ON | Low oxygen alert

Wokwi notes: use LDR to simulate finger placement, covering it dark gives low HR/SpO2 and triggers the alert, bright LDR simulates healthy readings. The 5-reading rolling average makes transitions smooth rather than jumpy.

Q48. Design Challenge, IoT for Jammu Smart City

Jammu, the winter capital of J&K, is on the cusp of a digital transformation. As the government pushes the Smart City Mission forward, IoT-enabled infrastructure can dramatically improve daily life for Jammu's residents, from cleaner air to unclogged traffic. I propose a monitoring and management system for three high-impact urban areas: street lighting, air quality, and waste bin fill levels. These were chosen because they directly affect citizen quality of life and have proven, low-cost IoT solutions.

System 1, Smart Street Lighting

The problem: Jammu's street lights often stay on throughout the day and turn off exactly when needed most, late at night, due to timer misconfiguration or fuse failures. This wastes electricity and leaves dark pockets that affect public safety.

Proposed solution: deploy LDR sensors on each street light pole. When ambient light drops below a threshold, the light turns on automatically. A PIR motion sensor is added to detect pedestrian activity, if no motion is detected for 10 minutes after midnight, lights dim to 40% brightness to save power.

Hardware per node: NodeMCU ESP8266 for Wi-Fi connectivity, LDR to detect ambient daylight, PIR sensor to detect pedestrian presence, relay module to control the streetlight circuit.

Data flow: LDR + PIR sensor to ESP8266 to MQTT to AWS IoT Core to Grafana dashboard. City engineers see a live map showing which lights are on or off, any failures, and power consumption per zone.

Estimated cost per node: 480 rupees (ESP8266 180, LDR 10, PIR 60, relay 80, PCB and housing 150).

System 2, Air Quality Monitoring

The problem: Jammu faces serious air pollution, especially in winters when biomass burning, vehicle emissions, and temperature inversions trap pollutants near ground level. Without real-time data the city can't issue health advisories or take targeted action.

Proposed solution: deploy MQ-135 gas sensor nodes at key intersections and parks. The sensor measures CO2, NH3, NOx, alcohol, benzene and smoke. Each node reports PM2.5-equivalent readings every 5 minutes and triggers an alert if levels exceed safe thresholds, AQI above 150.

Hardware per node: ESP32 for Wi-Fi and BLE, MQ-135 multi-gas sensor, DHT22 for temperature and humidity which affects AQI calculation, solar panel and Li-Ion battery for self-powered outdoor operation.

Data flow: MQ-135 + DHT22 to ESP32 to HTTPS POST to Azure IoT Hub to Power BI dashboard to SMS alert. Citizens can view a real-time AQI map via a city web portal, and alerts go to the municipal corporation when AQI crosses 200 in any zone.

Estimated cost per node: 620 rupees (ESP32 250, MQ-135 80, DHT22 90, solar and battery 200).

System 3, Smart Waste Bin Fill Level Monitor

The problem: municipal garbage trucks in Jammu follow fixed routes on fixed schedules, collecting from bins whether full or nearly empty. This wastes fuel and labor and causes overflow in high-traffic areas like Raghunath Bazaar and Canal Road.

Proposed solution: mount an HC-SR04 ultrasonic sensor inside each large waste bin to measure distance from lid to waste surface. When a bin is more than 80% full, it's flagged for collection, and the fleet management system dispatches trucks only to bins that need emptying, dynamic routing.

Hardware per node: ESP8266 as a low-cost Wi-Fi controller, HC-SR04 to measure fill distance, 18650 Li-Ion battery for weeks of operation per charge, waterproof enclosure to protect electronics outdoors.

Data flow: HC-SR04 to ESP8266 to MQTT to Google Firebase to fleet management app. Drivers see a phone app showing only bins that are 80%+ full, improving collection efficiency by an estimated 35-40%.

Estimated cost per node: 750 rupees (ESP8266 180, HC-SR04 70, battery and charger 250, enclosure 250).

Consolidated cost summary

System | Nodes (Pilot) | Cost per Node | Pilot Budget

Street Lighting | 200 lights | 480 | 96,000

Air Quality | 20 intersections | 620 | 12,400

Waste Bins | 100 bins | 750 | 75,000

Total | 320 nodes | - | 1,83,400

For a city of Jammu's scale, roughly 6.5 lakh population, a full deployment across all wards would cost approximately 25-40 lakh rupees, a fraction of the annual savings in electricity and municipal fleet costs.

Conclusion: smart city IoT isn't just about technology, it's about making governance efficient and life better. The three systems proposed, street lighting, air quality, and waste management, have quick ROI, proven technology, and can be deployed incrementally. Starting with a pilot in one ward, results can be measured and the system scaled city-wide. Jammu is ready for this transformation, and IoT is the tool to make it happen.

Q49. Security Vulnerability Analysis

Below is a poorly written ESP8266 IoT code snippet that connects to a public Wi-Fi network, collects sensor data, and publishes it to an MQTT broker. Five security vulnerabilities are identified below with corrected code for two of them.

Vulnerable code

#include <ESP8266WiFi.h>

#include <PubSubClient.h>

const char* ssid     = "PublicWiFi";

const char* password = "wifi12345";

const char* broker   = "broker.hivemq.com";

const char* topic    = "iot/sensor/data";

WiFiClient wifiClient;

PubSubClient client(wifiClient);

void setup() {

WiFi.begin(ssid, password);

client.setServer(broker, 1883);

client.connect("ESP8266Client");

}

void loop() {

float temp = analogRead(A0) * 0.1;

String payload = "temp=" + String(temp);

client.publish(topic, payload.c_str());

client.subscribe("iot/commands");

delay(5000);

}

Vulnerability 1, hardcoded Wi-Fi credentials in source code

const char* password = "wifi12345"; When this code is pushed to a GitHub repository, even a private one that could accidentally be made public, the Wi-Fi password is exposed for anyone to read. Automated bots constantly scan GitHub for leaked credentials. An attacker who finds this can join the Wi-Fi network without permission, intercept traffic from all devices on it, perform man-in-the-middle attacks on the IoT sensors, or use the connection for illegal activity. If the password is reused elsewhere, which is common, those accounts become vulnerable too. Severity: high.

Vulnerability 2, connecting to a public or unsecured Wi-Fi network

Public Wi-Fi has no encryption between device and router. Any other device on that network can sniff traffic with tools like Wireshark, intercept MQTT messages from the sensor, or perform an evil twin attack, setting up a hotspot with the same SSID "PublicWiFi" so the ESP8266 automatically connects to it, routing all data through the attacker's machine. Severity: high.

Vulnerability 3, unauthenticated MQTT broker on unencrypted port 1883

client.setServer(broker, 1883); with no username or password. The public HiveMQ broker on port 1883 uses no encryption and no authentication. Anyone on the internet can subscribe to the topic and read all sensor data, or publish fake data to corrupt the system. For a water level monitor or health device this is a serious privacy and integrity issue, since port 1883 sends everything in plaintext, readable by any network observer. Severity: critical.

Vulnerability 4, sensitive data published with no encryption or validation

client.publish(topic, payload.c_str()); sends sensor data as a plain, unvalidated string with no integrity check. An attacker who intercepts or injects messages can modify readings before they reach the cloud, causing wrong alerts or decisions, inject arbitrary payloads, or replay old readings to fool the system. For safety-critical IoT like flood detection or health monitors, false data is dangerous. Severity: medium-high.

Vulnerability 5, blind execution of incoming MQTT commands

The callback blindly executes incoming command strings, for example system(receivedCommand). This is a remote code execution vulnerability, arguably the worst in embedded security. Since the broker is public and unauthenticated (vulnerability 3), anyone on the internet can publish a command to iot/commands and the device executes it without validation. An attacker could reset the device, change Wi-Fi credentials, disable sensors, or in extreme cases take full control and turn the device into a bot for DDoS attacks, similar to the Mirai botnet. Severity: critical.

Corrected code for vulnerability 1, move credentials to config.h (gitignored)

#ifndef CONFIG_H

#define CONFIG_H

#define WIFI_SSID      "YourHomeWiFi"

#define WIFI_PASSWORD  "YourSecurePassword"

#define MQTT_USER      "your_mqtt_username"

#define MQTT_PASS      "your_mqtt_password"

#endif

.gitignore

config.h

Updated main code

#include "config.h"

void setup() {

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

}

Corrected code for vulnerability 3, encrypted MQTT (port 8883 with TLS) plus authentication

#include <ESP8266WiFi.h>

#include <WiFiClientSecure.h>

#include <PubSubClient.h>

#include "config.h"

WiFiClientSecure secureClient;

PubSubClient mqttClient(secureClient);

void setup() {

Serial.begin(115200);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) delay(500);

secureClient.setInsecure();

mqttClient.setServer("broker.hivemq.com", 8883);

reconnect();

}

void reconnect() {

while (!mqttClient.connected()) {

if (mqttClient.connect("ESP8266Client_UniqueID", MQTT_USER, MQTT_PASS)) {

Serial.println("MQTT Connected securely.");

} else {

Serial.printf("MQTT failed, rc=%d. Retrying in 5s...\n", mqttClient.state());

delay(5000);

}

}

}

void loop() {

if (!mqttClient.connected()) reconnect();

mqttClient.loop();

float temp = analogRead(A0) * 0.1;

String payload = "{\"temp\":" + String(temp) + "}";

mqttClient.publish("iot/sensor/data", payload.c_str());

delay(5000);

}

What changed: WiFiClientSecure instead of WiFiClient for TLS encryption, port 8883 instead of 1883 for encrypted MQTT, MQTT_USER and MQTT_PASS from config.h for an authenticated connection, and credentials loaded from a gitignored file rather than hardcoded.

Summary table

# | Vulnerability | Severity | Fix

1 | Hardcoded Wi-Fi password in source | HIGH | Move to config.h, add to .gitignore

2 | Connecting to public/open Wi-Fi | HIGH | Use WPA2/WPA3 private network

3 | Unauthenticated MQTT on port 1883 | CRITICAL | Use port 8883 + TLS + credentials

4 | Unencrypted/unvalidated data publish | MEDIUM | Use TLS + JSON schema validation

5 | Blind execution of incoming commands | CRITICAL | Validate, authenticate, and whitelist all commands

Q50. Reflection and Learning Portfolio

What I built

This assignment pushed me to go beyond theory and actually build things that work. Across the three modules I worked on a range of projects, from fetching live weather data via REST APIs and comparing it with a physical DHT11 sensor, to designing a full flood-detection node using an HC-SR04 ultrasonic sensor with rolling averages and a live Chart.js dashboard. I also built the logic for a smart door lock that generates and sends OTPs via Bluetooth, and a greenhouse controller that manages three separate relay outputs using bang-bang control with hysteresis.

Each project forced me to confront a different challenge. The weather API project taught me that theory about APIs and JSON parsing is one thing, but when parsing fails at 2 AM because of a missing field in the response, you learn to read documentation carefully. The greenhouse controller made me appreciate why delay() is dangerous in real systems, once I switched to millis()-based timing my LCD updated smoothly without blocking the sensor reads.

What I learned

The biggest lesson from this module is that real IoT is about reliability, not just functionality. It's easy to make something work once. Making it work consistently, at 3 AM, when the Wi-Fi drops, when the sensor gives a noisy reading, or when the broker goes down, that's the real challenge.

I learned to use rolling averages to filter noisy sensor data, which made a real difference in the ultrasonic readings for the flood detection project. I learned why MQTT is preferred over HTTP for IoT, not just as a memorized fact but because I saw the overhead difference firsthand when I tried a quick HTTP polling approach first and it clearly wasn't practical for constrained devices.

Security was another eye-opener. Writing the vulnerability analysis for Q49 made me realize how casually I had been treating credentials in my own test code. Seeing how trivially a public GitHub repo can expose a Wi-Fi password changed how I think about every line I write.

Git also became more than just a way to save code. The discipline of meaningful commits, writing clear messages and committing after each feature rather than dumping everything at the end, made it much easier to trace back when something broke.

Struggles that taught me the most

Getting JSON parsing right in the weather API project genuinely took me hours. ArduinoJson is powerful but unforgiving, a wrong document size or a missing const char* cast and nothing works. I learned to use the ArduinoJson assistant tool online, and that habit of checking documentation before assuming will stay with me.

The OTP door lock project was frustrating initially because I kept using delay() for the 30-second countdown timer, which froze the keypad input. Switching to millis() was a revelation, the system stayed responsive while counting down in the background. That one change made the entire project feel professional.

How I'll apply IoT in my future

I'm studying engineering, and I see IoT as the bridge between the physical world and intelligent systems. The skills I built here, sensor interfacing, protocol selection, data flow design, and security thinking, are directly applicable whether I go into embedded systems, smart infrastructure, or product development.

In my region, J&K, there are genuine unsolved problems in agriculture, disaster management, and urban infrastructure where affordable IoT deployments could make a real difference. The Jammu Smart City proposal I designed for Q48 felt personal because these are problems I actually see around me.

My next IoT project, Smart River Pollution Monitor for Tawi River

The Tawi River runs through Jammu, and industrial and domestic waste discharge is a growing problem. My next IoT project would be a distributed water quality monitoring network for the Tawi.

Each node would include a pH sensor for acidity, a TDS sensor for contamination level, a DS18B20 for water temperature, and a turbidity sensor for visual clarity. It would run on an ESP32 for Wi-Fi and deep sleep capability for battery operation, and connect via a LoRa mesh network since riverbanks have poor cellular coverage, routing data to a gateway at the riverbank police station or municipality office. Data would flow over MQTT to AWS IoT Core, stored in DynamoDB, and visualized on a public Grafana dashboard. If pH drops below 6.5 or TDS exceeds safe limits, a Telegram bot would alert the J&K Pollution Control Board automatically.

The cost per node would be approximately 1,200-1,500 rupees, and a 10-node pilot across 5 km of the river would give real-time data that's currently unavailable to authorities. This project feels meaningful because it isn't just an exercise, it's a real problem with real data that could genuinely help. That's what I want to build: IoT systems that matter.